## Feature Categories Overview

Our feature taxonomy classifies all input data into four categories based on their impact on model performance and the effort required to obtain them.

Must Have Features (Essential – Model Won’t Function Without These)

These features are absolutely required for the models to work. Without them, the approach fails.

|  | Feature | Why Its Important | Source/Instrument | Typical Units |
| --- | --- | --- | --- | --- |
| 1 | Production Date (DATEPRD) | Defines time axis for survival analysis (WB); chronological anchor for time-series modeling (WP); essential for capturing production lifecycle phases | SCADA timestamp logging | YYYY-MM-DD |
| 2 | Average Bottomhole Pressure (AVG_DOWNHOLE_PRESSURE) (sometimes called Average Flowing Bottomhole Pressure) | Most critical predictor - directly reflects reservoir energy and drive mechanism strength; governs critical drawdown threshold (WB); controls total liquid productivity (WP) | Permanent Downhole Gauge (PDG) Calculated Methods (When PDG Not Available): Wellhead Pressure + Hydrostatic Column | psi or bar |
| 3 | Pressure Drawdown (ΔP) | Primary physical driver of coning (WB); exceeding critical drawdown triggers breakthrough; affects total liquid rate (WP) | Calculated: Static reservoir pressure (psi) – Flowing bottomhole pressure (AVG_DOWNHOLE_PRESSURE) | psi |
| 4 | Oil Column Thickness or Oil Column Height (h) | Distance to oil-water contact determines coning susceptibility; the dominant factor for breakthrough timing; small h values dramatically increase risk | Method 1: Direct Measurement (Resistivity logs (deep induction, laterolog) from initial well logging) Method 2: Seismic + Well Control (Source: 3D seismic interpretation + structural mapping ) Method 3: Pressure Gradient Analysis (Source: MDT pressure points at multiple depths ) | ft or m |
| 5 | Cumulative Oil Production or Cumulative Production (Np) | The total volume of oil produced from a well from first production to the current date. Primary depletion indicator for survival analysis (WB); depletion metric post-breakthrough (WP); tracks reservoir maturity | Daily production volumes (BORE_OIL_VOL) | bbl |
| 6 | Production Lifetime or Well Age in Production or Days Since First Production | The number of calendar days elapsed from the date of first commercial production to the current date. Time variable for survival models (WB); normalizes production lifecycle across wells | Calculated from DATEPRD | Days |
| 7 | Absolute Permeability or Formation Permeability Permeability (k) | A measure of the rock's ability to transmit fluids. Quantifies how easily fluids flow through the pore network when subjected to a pressure gradient. Controls flow velocity and pressure distribution (WB); high k increases coning tendency; flow capacity affects water mobility (WP) | Method 1: Pressure Transient Analysis (Source: Pressure buildup or drawdown tests. Tools: Schlumberger Saphir, Kappa Ecrin software) Method 2: Core Analysis (Source: Core samples from drilling. Instrument: Core flooding apparatus (gas or liquid permeameter) Method 3: Well Log Correlations (Source: Porosity and water saturation logs) Method 4: Production Data Analysis (PDA) (Source: Historical production + pressure data. Software: Topaze, Harmony RTA) | mD or Darcy |
| 8 | Water Cut or Water Fraction (WC) | The ratio of water production to total liquid (oil + water) production Defines breakthrough event threshold (WC > 2%) for WB; direct transformation of water/oil ratio for WP; fractional flow physics | Calculated: WC = BORE_WAT_VOL / (BORE_OIL_VOL + BORE_WAT_VOL) Test Separator or Multiphase Flow Meter or BS&W Analysis | Fraction (0-1) |
| 9 | Well Trajectory Classification or Well Type (Vertical/Horizontal) | Fundamentally alters coning geometry (WB); horizontal wells delay breakthrough significantly; drainage pattern affects water production profile (WP) | NPD registry / Well completion records Categories: Vertical Well, Deviated Well, Horizontal Well, Multilateral Well | Categorical |
| 10 | Daily Oil Production Volume at Bore (Reservoir) Conditions or Oil Volume (BORE_OIL_VOL) | Core production metric; defines total liquid rate (WP); production rate influences drawdown (WB) | Separator metering / test separator or Multiphase Flow Meter or Tank Gauging + Allocation | bbl/day or m³/day |
| 11 | Daily Water Production Volume at Bore (Separator) Conditions or Water Volume (BORE_WAT_VOL) | The volume of water produced per day, measured after separation from oil and gas at surface conditions. Includes formation water, injected water (in waterfloods), and condensed water. Target variable for forecasting; time-series models require historical water rates; defines water production behavior post-breakthrough | Water cut meter + Total liquid or BS&W calculation or Test Separator | bbl/day or m³/day |
| 12 | Well identifier or Well Bore Code | Identifies wells, enables grouping and multi-well modeling; critical for well-specific behavior capture and transfer learning | Well master data | String ID |
| 13 | Daily Water Injection Volume (BORE_WI_VOL) | Direct driver in waterflooded systems (WB); sweep efficiency affects breakthrough; direct source term (WP); injection-production correlation critical | Injection flow meters Location: Injection wellhead or manifold | bbl/day or m³/day |
| 14 | Water Cut Rate of Change or WC Velocity (ΔWC/Δt) | Rate of water cut increase; predicts acceleration in water production; captures speed of water advancement | Numerical differentiation of water cut | %/day |
| 15 | GOR (Gas-Oil Ratio) | Indicator of reservoir pressure maintenance (WB); declining GOR signals energy loss and often precedes increasing water production (WP) | Calculated: GAS_VOL / OIL_VOL | scf/bbl or Sm³/Sm³ |
| 16 | Rate Change (dq/dt) | Captures operational interventions; rate changes affect water influx dynamics; sudden changes may precede breakthrough acceleration | q(t) – q(t-1) | bbl/day² |
| 17 | On-Stream Hours | Distinguishes producing vs. shut-in periods (WB); affects cumulative stress on reservoir; production uptime affects cumulative water (WP) | Flow computer / SCADA counters | Hours (0-24) |
| 18 | Average Choke Size (or Average Choke Bean Size) AVG_CHOKE_SIZE_P | Operational control affecting drawdown (WB); choke-up events can trigger breakthrough; choke management influences water production (WP) | Choke position transmitter or Stem position indicator (ZT-001 or similar SCADA tag) | 64ths inch or % |
| 19 | Reservoir Pressure Decline Rate or Pressure Depletion Rate (dP/dt) | Captures dynamic pressure decline (WB); rapid depletion correlates with breakthrough; dynamic reservoir energy indicator (WP) | Numerical differentiation of pressure time series | psi/day or bar/day |
| 20 | Mobility Ratio (M) | Determines frontal stability (WB); M > 1 favors water breakthrough; governs water-oil displacement efficiency (WP); M > 1 accelerates production growth | Derived: (krw/μw) / (kro/μo) | Dimensionless |
| 21 | Porosity (φ) | Affects depletion rate and pressure decline (WB); lower importance than permeability for breakthrough timing; storage capacity affects depletion timescale (WP) | Well logs (Density-Neutron, Sonic) or core analysis | Fraction (0-1) or % |
| 22 | Water Saturation (Sw) | Initial condition affecting mobile water volume (WB); critical in transition zones; initial mobile water impact diminishes post-breakthrough (WP) | Resistivity logs (Archie equation) | Fraction (0-1) or % |
| 23 | Perforation Interval Ratio (hpo/ho) | Determines exposure to water zone (WB); higher ratio accelerates breakthrough; influences drainage geometry (WP) but effect is static | Completion diagram | Fraction (0-1) |
| 24 | Fluid Viscosity (μ) | Component of mobility ratio (WB); high oil viscosity increases breakthrough risk; affects mobility (WP) but usually stable unless heavy oil | PVT lab data at reservoir P&T | cp or mPa·s |
| 25 | Gas Volume (BORE_GAS_VOL) | Used to calculate GOR trends which diagnose reservoir issues (WB); provides same information as GOR more directly (WP) | Gas meter (orifice, turbine, ultrasonic) | Mscf/day |

Nice to Have Features (Marginal Performance Gains)

These features provide incremental improvements but require additional data sources.

| Feature | Why Its Important | Source/Instrument | Typical Units |
| --- | --- | --- | --- |
| True Vertical Depth (TVD) | Proxy for pressure gradient and OWC proximity (WB); less direct than oil column height; static well property with minimal forecasting contribution (WP) | Directional survey | Ft or m |
| Relative Permeability (kr) | Governs fractional flow curves (WB); difficult to obtain routinely; useful for physics-informed ML (WP) but rarely available | Core flooding experiments | Fraction (0-1) |
| Avg Wellhead Pressure (AVG_WHP_P) | The average pressure measured at the surface of the well, specifically at the top of the production tubing (wellhead), over a 24-hour period. Surface pressure; less direct indicator than downhole pressure (WB); used with downhole to compute drawdown; surface constraint (WP) | Wellhead pressure transmitter (PT-001) (SCADA tag typically: PT-001 or similar) Measured at the tubing head/Christmas tree, typically 3-10 feet above ground level at the wellhead assembly. | psi or bar |
| Tortuosity | Wellbore complexity; minor effect compared to well type (WB); wellbore characteristic with negligible effect on rates (WP) | Calculated: MD / TVD | Dimensionless ratio |
| Matrix/Fracture Permeability | Critical in naturally fractured reservoirs (WB); limited applicability in conventional settings; affects water sweep patterns in fractured systems (WP) | DPDK simulation models or special tests | mD |
| Matrix/Fracture Porosity | Dual-porosity systems only (WB); complex to characterize; naturally fractured reservoirs only (WP); high characterization uncertainty | Core + imaging logs | Fraction (0-1) or % |
| Oil Gravity (°API) | Affects viscosity (WB); secondary to direct viscosity measurement; relatively constant property with minimal forecasting utility (WP) | Production stream analysis or wellhead sample | °API or specific gravity |
| Flow Kind (Artificial Lift) | Affects drawdown management (WB); indirect effect on breakthrough timing; affects operating envelope (WP); lift changes alter water handling | Operations data / NPD measurement protocol | Categorical |

Can Have Features

| Feature | Why Its Important | Source/Instrument | Typical Units |
| --- | --- | --- | --- |
| Avg Downhole Temperature | Secondary PVT effect (WB); temperature variation is small in most reservoirs; minor PVT effect (WP); temperature changes minimal in most scenarios | Distributed Temperature Sensing (DTS) / Downhole thermocouples | °F or °C |
| Avg Wellhead Temperature | Minimal direct impact on breakthrough physics (WB); surface measurement with minimal forecasting value (WP) | Surface temperature sensors (TT-001) | °F or °C |
| Condensate Gas Ratio (CGR) | Gas condensate wells only; not applicable to oil wells; highly specialized application | Calculated: CONDENSATE_VOL / GAS_VOL | bbl/MMscf |