Data Requirements vs Data Received

Gap Analysis Checklist

## Source files reviewed

- STIXOR_Sharing_Data.xlsx — Production data (Rates + Pressures sheets, 6,652 well-months, Mar 1978 – Jun 2025)

- Subsurface data — Stixors Technologies.xlsx — Static rock & completion properties (22 wells, 8 fields)

## Summary

| Status | Count | Meaning |
| --- | --- | --- |
| OK | 10 | Received and usable as-is |
| PARTIAL / CLARIFY | 9 | Received with gaps, or needs verbal confirmation |
| MISSING | 10 | Not provided — must be requested |
| N/A | 2 | Not applicable to a gas field on natural depletion |

All 22 wells have production history plus core static rock properties (porosity, permeability, Sw, net pay, perforations, chlorides). The 10 missing items below should be formally requested at the Apr 8 meeting — items #20–23 (coordinates, TVD, GWC depth) are the highest priority and unblock the physics-informed feature set.

## Detailed checklist

| # | Category | Field requested | Status of received data | Status | Action / Question for Mari |
| --- | --- | --- | --- | --- | --- |
| 1 | Production rates | Monthly gas volume (MMcf) per well | Provided in Rates sheet, full history Mar 1978 – Jun 2025 | OK | — |
| 2 | Production rates | Monthly water volume (bbl) per well | Provided in Rates sheet, full history | OK | — |
| 3 | Production rates | Monthly oil volume per well | Not provided (gas field — likely not applicable) | N/A | Confirm there is no oil/condensate phase being produced |
| 4 | Operational | Days on production per month | Provided in Rates sheet | OK | Confirm column is reliable across full history (not back-filled) |
| 5 | Operational | Choke setting (1/64") | PARTIAL → effectively MISSING for pre-2015 20–47% coverage, only the last 10 years. M-11 has choke from 2015 onwards but nothing for the prior 37 years of production. Same for M-13, M-22, M-41, M-50, M-51, etc. | MISSING | Confirm whether pre-Aug 2015 choke records exist in archives for the 17 legacy wells. If not, we'll either drop choke or impute and document the caveat. |
| 6 | Operational | Water injection rates (nearby wells) | Not provided | N/A | Confirm field is on natural depletion only — no water injection / pressure support anywhere |
| 7 | Pressure | Wellhead Flowing Pressure (WHFP, psig) | Provided from Jun 1990 onwards (5,354 records) Monthly Average/ [To be checked] | PARTIAL | Pre-1990 data missing for M-11 and M-13. Any archived shut-in or build-up data from 1978–1990? |
| 8 | Pressure | Line Pressure (psig) High pressure might affect the production of that month Same choke size but lesser production | Provided but ~62% null Monthly Average/backfill imputations | PARTIAL | Confirm whether sparse coverage is real or a recording gap |
| 9 | Pressure | Bottomhole Pressure (BHP) measurements | Not provided Random estimations [To be checked] | MISSING | Share any BHP surveys / annual shut-in measurements available in PTA reports |
| 10 | Pressure | Initial reservoir pressure per well | Not provided Random estimations [To be checked] | MISSING | Share initial reservoir pressure (P_i) per well at start of production |
| 11 | Rock properties | Porosity (φ) | Provided for all 22 wells (range 0.17–0.24) | OK | — |
| 12 | Rock properties | Permeability (k, mD) | Provided for all 22 wells (PTA-derived, range 3.3–37 mD) | OK | — |
| 13 | Rock properties | Water saturation (Sw) | Provided for all 22 wells (range 0.13–0.46) | OK | — |
| 14 | Rock properties | Net pay thickness | Provided for all 22 wells, in measured depth (MD) TVD and MD are same for all vertical wells Net pay 500m for horizontal wells perforated horizontally 10-10.5m net pay Average of other vertical wells | PARTIAL | Share net pay in true vertical depth (TVD) where it differs from MD, especially for horizontals |
| 15 | Rock properties | Skin factor | Provided for 17 of 22 wells (PTA-derived) Rock damage due to drilling Not a reservoir property, induced parameter, fudge factor (can change over time) Decide and have concensus on using or not using it | PARTIAL | Acceptable — new horizontals (M-122H–126H) naturally lack PTA. No action. |
| 16 | Rock properties | Net-to-Gross (NTG) | Not provided | MISSING | Nice-to-have. Share if available, otherwise we use net pay alone. |
| 17 | Completion | Top perforation depth Start and end of the performation | Provided for all 22 wells (in MD) | PARTIAL | Share Top Perf in TVD, especially for the 5 horizontal wells |
| 18 | Completion | Bottom perforation depth | Provided for all 22 wells (in MD) | PARTIAL | Share Bottom Perf in TVD for the 5 horizontal wells |
| 19 | Completion | Completion type (open hole, slotted liner, gravel pack, frac'd) Horizontal have slotted liner completions Vertical have performations in the casing | Not provided | MISSING | Share completion type per well — likely explains why M-122H broke through fast while M-123H–126H are still dry Extended surface area in contact with the reservoir |
| 20 | Geometry | Well surface coordinates (X/Y or lat-long) | Not provided Spacing between the wells Different corrdinate systems (WGS 84) to use the corrdinates Vertical wells (bottom hole corrdinates) Horizontal wells (surface and bottom points) Area in contact (trajecttory or design of the well) | MISSING | HIGHEST PRIORITY. Needed for neighbour features, distance-to-OWC, and analog matching for new horizontals |
| 21 | Geometry | Bottomhole coordinates (for horizontals) | Not provided | MISSING | Share for the 5 horizontal wells (M-122H–126H) |
| 22 | Geometry | True Vertical Depth (TVD) Trajectory | Not provided | MISSING | Share TVD per well — coning physics is built around vertical distance from perforations to the water level |
| 23 | Geology | Gas-Water Contact (GWC) depth Constant for the reservoir | Not provided | MISSING | Single most important input for water coning prediction. Even one number per reservoir is fine. |
| 24 | Geology | Reservoir formation per well | Implied (HRL suffix), but new horizontals have very different perf depths and lengths | CLARIFY | Confirm all 22 wells produce from Habib Rahi Limestone, or whether the new horizontals (M-122H–126H) target a deeper zone (Pirkoh / Sui Main Limestone) |
| 25 | Geology | Formation tops / structure maps | Not provided | MISSING | Nice-to-have. Useful for spatial analog matching. |
| 26 | Fluid | Chlorides (formation water salinity, ppm) | Provided for all 22 wells (1,215–23,000 ppm) Anything <1000 is not a dangerous number to have 10-20% variability | OK | — |
| 27 | Fluid | PVT properties (gas Z-factor, condensate yield, water viscosity Dry gas z factor -> gas composition Expansion factor Viscosity not needed for gas No condensate Gas gravity -> calculate z factor | Not provided | MISSING | Nice-to-have for material-balance / p-z diagnostics. Not critical. |
| 28 | Event label | Engineering 'first water' date per well If chlorides is higher then the breakthrough Sudden increase in WGR - WGR 2) Chlorides | Not provided (currently inferred from WGR > 5 bbl/MMcf threshold) | MISSING | Share engineering judgment date for first true water breakthrough per wet well — replaces our inferred label |
| 29 | Data quality | M-51 immediate water (month 0) | Shows water from very first month | CLARIFY | Was this completion flowback or genuine formation water from day one? Likely needs to be excluded. |
| 30 | Data quality | Always-dry wells (M-13, M-22, M-57, M-E-2) Dry test for these wells | Zero water across full history | CLARIFY | Confirm these are genuinely dry, not a measurement gap. These are our most valuable censored wells. |

## Top 5 priority asks

- Well coordinates (surface X/Y + bottomhole for horizontals) — item #20–21

- True Vertical Depth (TVD) per well + perforations in TVD — item #14, 17, 18, 22

- Gas-Water Contact (GWC) depth — item #23

- Initial reservoir pressure + any BHP surveys — item #9, 10

- Engineering 'first water' date per well + completion type — item #19, 28